Avaya CallPilot® 1005r System Evaluation

for

## Customer ______________________________

SUMMARY:   A system evaluation of the Customer ______________________ Avaya CallPilot 1005r system  was requested

by  __________________________The evaluation was performed on _________.  The nature of the evaluation

was to determine if the CallPilot 1005r system was installed per Avaya manufacturing

specifications and Product Bulletin requirements.

DISTRIBUTION:

EVALUATED BY:

DATE:

| Document Number | NN44200-316 |
| --- | --- |
| Document Status | Standard |
| Document Version | 01.03 |
| Date | December 2010 |

## SAMPLE SITE LAYOUT

1005r connected to Avaya Communication Server 1000

.

1005r connected to Meridian 1

## .

Location Profile

### Site Information:

| Engineer: |  | Evaluation Date: |  |
| --- | --- | --- | --- |
| Distributor: |  | Customer: |  |
| Address: |  | Address: |  |
| Contact: |  | Site Telephone: |  |
| Telephone: |  | Attendees: |  |
| Email: |  |  |  |
| GNTS Case# |  |  |  |

### System Information:

| System Serial Number: |  |
| --- | --- |
| Number of Channels |  |

|  | Max |  | Used | Comments |
| --- | --- | --- | --- | --- |
| Voice Message Seats |  |  |  | These values can be found by going to CallPilot Manager -> System -> Server Settings. View Server settings for Resources. NOTE: If any of the “Used” values are close to the “Max” values, the customer should be advised. |
| Fax Message Seats |  |  |  |  |
| Desktop Message Seats |  |  |  |  |
| E-Mail By Phone (TTS) Message Seats |  |  |  |  |
| Automated Speech Recognition Seats |  |  |  |  |
| Storage Hours | 2400 |  |  |  |

FINDINGS AND RECOMMENDATIONS

Introduction:

### Discrepancies and Recommendations:

| Item #18 [Major] Sample Discrepancy Findings: (Priority Item) Recommendation: | ELAN Packet loss degrading CallPilot performance. ELAN connected through layer 2 switch. Switch dropping packets. Replace Layer 2 switch. |
| --- | --- |

| Item # Findings: Recommendation: |  |
| --- | --- |

| Item # Findings: Recommendation: |  |
| --- | --- |

| Item # Findings: Recommendation: |  |
| --- | --- |

Conclusion:

This report is based on checklist items contained in this document. The checklist item under each subheading is answered with a “Y” or “N”, signifying that it either complies or does not comply with Avaya specifications. An “N/A” means that the checklist question does not apply in this instance. The specifications are based on Avaya Practices, Product Bulletins, Product Advisories, and General Release Bulletins. Each checklist item is given a weight. The item may be deemed as “Critical, Major, Minor, or Recommended” in nature. A system evaluation is found to be “non-compliant” when one “Critical” or two “Major” discrepancies have been identified. Checklist weighting is not given to Applications products questions. The aim of an evaluation is to ensure installation completeness, optimize system performance/reliability, and provide a safe environment for personnel.

- Check the appropriate entry

| The system installed at this location complies with Avaya system evaluation guidelines |  |
| --- | --- |

| The system installed at this location does not comply with Avaya system evaluation guidelines. When the identified discrepancies are corrected, the system will be considered “compliant”. Make further comments here as well |  |
| --- | --- |

# CHECKLIST

##### Hardware Installation Guide 		 NN44200-308

Maintenance and Diagnostics Guide	 NN44200-704

## Equipment Room Environment

| Item | Description | Meets Specifications |
| --- | --- | --- |
|  | The CallPilot server should be installed in a safe, clean, well-ventilated, and easily accessible location in the equipment room. The server should be kept away from equipment that produces strong magnetic fields. [Critical] | ________________ |
|  | Equipment room temperature is between 5°C and 35°C (41°F to 95°F) [Critical] | ________________ |

Power and Grounding

|  | The CallPilot server should derive AC power & grounding from the same source as the host PBX. One 120 VAC 15 amp breaker is required for each server power supply and associated ancillary devices (modem, terminal, etc.). [Critical] | ________________ |
| --- | --- | --- |
|  | Ensure the CallPilot server chassis and equipment racks are isolated from other foreign sources of ground. Acceptable isolation methods include: isolation pads, grommeted washers, chassis side rail strips, and non-conducting washers.[Critical] | ________________ |
|  | Ensure ESD procedures are being adhered to.[Critical] | ________________ |
|  | If other equipment is also installed in the same 19" rack, ensure that all equipment derives ground from the same service panel as CallPilot and the switch, whether or not the equipment is AC- or DC-powered.[Critical] | ________________ |
|  | UPS Requirements | ________________ |
|  | If a UPS is providing AC power and grounding for the CallPilot server, then the UPS system, in turn, must be powered and grounded from the Avaya Communication Server 1000 or Meridian 1 AC Service Panel. [Critical] | ________________ |

General Installation

| Item | Description | Meets Specifications |
| --- | --- | --- |
|  | Cables associated with the CallPilot server must be installed in an orderly fashion, labeled on both ends, and fastened down completely on both ends. (This may minimize troubleshooting problems and tripping hazards.) [Major] | ________________ |
|  | Ferrite Cores for the triple DS30X I/O are installed at each end of the cable[Critical] (These are factory installed, but could be inadvertently removed) | ________________ |
|  | Ensure there are no unauthorized connections to the ELAN via the hub or switch. The ELAN carries call information between CallPilot and the switch. Any other network traffic could degrade CallPilot performance. [Major] | ________________ |
|  | The ELAN and CLAN are on different subnets. [Critical] | ________________ |
|  | ELAN cables from the ELAN hub/switch to the PBX CORE 0 and CORE 1 I/O panel connectors are installed. | ________________ |
|  | The server has an operational mouse, terminal, and keyboard. .[Recommended] | ________________ |

|  | General Installation |  |
| --- | --- | --- |
| Item | Description | Meets Specifications |
|  | Server power supply LEDs, where applicable, are illuminated green in color. [Critical] | ________________ |
|  | No alarms are present on the front cover of the server[Critical] | ________________ |
|  | If ELAN is connected to switch via a layer 2 switch, ensure there is no packet loss. .[Major] | ________________ |
|  | All RAID drives are operational. (start ->programs -> MegaRaid Console -> All drives show green and online) [Critical] | ________________ |
|  | The front panel should be relatively free of dust.[Major] | ________________ |
|  | No critical CallPilot Manager alarms present. CallPilot Manager -> System ->Alarm Monitor[Critical] | ________________ |
|  | No critical CallPilot Manager events present. CallPilot Manager -> System ->Event Browse[Critical] | ________________ |
|  | All channels are in an “idle” or “active” state [Major] Maintenance Administration>Channel Monitor | ________________ |
|  | All DSPs are in an “idle” or “active” state [Major] Maintenance Administration>Multimedia Monitor | ________________ |
|  | An operational RAS modem is installed on the USB port of the server. [Major]. This modem is required for remote support. | ________________ |

|  | There are no mapped drives in the system. The CD-ROM drive is drive Z:[Major] | ________________ |
| --- | --- | --- |
|  | Server hard drive usage is below 90%[Critical] My Computer>Right mouse click on any drive>Go down to Properties | ________________ |
|  | Available memory not below 4 MBytes[Critical] Start -> Programs -> Administrative Programs -> Performance | ________________ |
|  | All Mgate cards connected to the same MPB96 cards must reside in the same Media Gateway/Media Gateway Expansion pair (Avaya CS 1000E only). For CS 1000E release 5, the Mgate cards can be placed in two separate Media Gateway shelves provided the MGC DECT Clock Reference cable (NTDW67AAE5) is installed between the two shelves. | ________________ |
|  | Processor time not more than 90% for any sustained period. [Critical] Start -> Programs -> Administrative Programs -> Performance (or Performance Monitor). | ________________ |

|  | Maintenance Items |  |
| --- | --- | --- |
| Item | Description | Meets Specifications |
|  | Verify that the most up-to-date level CallPilot Server PEPs, and CallPilot Manager are installed. Ensure the associated patches are on the Meridian 1 or CS 1000. A visit to the Avaya support web page is recommended to obtain the most current information. [Recommended] http://www.avaya.com/support | ________________ |
|  | Ensure that both full and partial backups have been completing successfully. [Major] Schedules for full and partial backups to tape must be configured. (system -> backup/restore) Tapes must be stored in a secure place away from magnetic fields. Tape backup and tape rotation logs should be administered also. | ________________ |
|  | All CallPilot CDs and DVDs are available in case of the requirement to re-install the system. | ________________ |
|  | Backups are not scheduled during “audits” (avoid 12 AM to 4AM) [Major] | ________________ |
|  | No “third party” software is installed and running on the server. [Major] (Symantec Corporate Edition 7.60/8.1, McAffee Enterprise 7.0/ NetShield 4.5, eTrust Antivirus 6.0/InnoculateIT 4.53, and ServerProtect 5.58 are okay. Do not schedule during high traffic or audits. See P-2003-0151 Version 3) | ________________ |
|  | Application Builder has no “locked” applications [Major] | ________________ |
|  | PcAnywhere or RDC is configured and in “ready” mode on the server. (Required for remote support).[Major] | ________________ |

|  | Administrator password is documented and available to the administrator (in a secure location). | ________________ |
| --- | --- | --- |
|  | PBX has all required Patches for CallPilot | ________________ |
|  | CallPilot software is compatible with CallPilot Manager and Application Builder | ________________ |
|  | ELAN and CLAN interfaces have static IP addresses | ________________ |
|  | ELAN interface does not have a default gateway | ________________ |
|  | Remote Notification works | ________________ |
|  | Check for DRWTSN32 and user.dmp logs. Make sure “Visual Notification” is not checked. | ________________ |
|  | No shared folders exist on the CallPilot server | ________________ |
|  | Observations/Comments: |  |