Avaya CallPilot® 202i System Evaluation

for

## Customer ______________________________

SUMMARY:   A system evaluation of the Customer ______________________ Avaya CallPilot 202i system  was requested

by  __________________________The evaluation was performed on _________.  The nature of the

evaluation was to determine if the CallPilot 202i system was installed per Avaya manufacturing

specifications and Product Bulletin requirements.

DISTRIBUTION:

EVALUATED BY:

DATE:

| Document Status | Standard |
| --- | --- |
| Document Number | NN44200-318 |
| Document Version | 01.02 |
| Date | December 2010 |

Sample Site Layout

Location Profile

### Site Information:

| Engineer: |  | Evaluation Date: |  |
| --- | --- | --- | --- |
| Distributor: Address: |  | Customer: Address: |  |
| Contact: |  | Site Telephone: |  |
| Telephone: |  | Attendees: |  |
| Email: |  |  |  |

### System Information:

| System Serial Number: |  |
| --- | --- |
| Number of Channels |  |

|  | Max |  | Used | Comments |
| --- | --- | --- | --- | --- |
| Voice Message Seats |  |  |  | These values can be found by going to CallPilot Manager -> System -> Server Settings. View Server settings for Resources. NOTE: If any of the “Used” values are close to the “Max” values, the customer should be advised. |
| Fax Message Seats |  |  |  |  |
| Desktop Message Seats |  |  |  |  |
| E-Mail By Phone (TTS) Message Seats |  |  |  |  |
| Automated Speech Recognition Seats |  |  |  |  |
| Storage Hours | 350 |  |  |  |

# FINDINGS AND RECOMMENDATIONS

### Discrepancies and Recommendations:

| Item # Findings: Recommendation: |  |
| --- | --- |

| Item # Findings: Recommendation: |  |
| --- | --- |

| Item # Findings: Recommendation: |  |
| --- | --- |

| Item # Findings: Recommendation: |  |
| --- | --- |

Conclusion

This report is based on checklist items contained in this document. The checklist item under each subheading is answered with a “Y” or “N”, signifying that it either complies or does not comply with Avaya specifications. An “N/A” means that the checklist question does not apply in this instance. The specifications are based on Avaya Practices, Product Bulletins, Product Advisories, and General Release Bulletins. Each checklist item is given a weight. The item may be deemed as “Critical, Major, Minor, or Recommended” in nature. A system evaluation is found to be “non-compliant” when one “Critical” or two “Major” discrepancies have been identified. Checklist weighting is not given to Applications products questions. The aim of an evaluation is to ensure installation completeness, optimize system performance/reliability, and provide a safe environment for personnel.

- Check the appropriate entry

| The system installed at this location complies with Avaya system evaluation guidelines |  |
| --- | --- |

| The system installed at this location does not comply with Avaya system evaluation guidelines. When the identified discrepancies are corrected, the system will be considered “compliant”. |  |
| --- | --- |

# CHECKLIST

## Equipment Room Environment

|  | 202i Installed in a Large Meridian 1 Switch. | Meets Specifications |
| --- | --- | --- |
|  | The Meridian 1 system has been audited and meets specifications. |  |
|  | NTRH3501 Cable is securely fastened to the three backplane connectors, and the IO panel |  |
|  | NTRH0912 Multi-IO Cable securely connected to the NTRH3501 connector at the IO panel, and the amphenol drain wire is connected to frame ground |  |
|  | 202i Installed in an Option 11C |  |
|  | The Option 11C system has been audited and meets specifications. |  |
|  | The NTRH1407 Intermediate SCSI cable is securely fastened to the SCSI connector on the faceplate of the 202i. |  |
|  | The NTRH1407 Intermediate SCSI cable bracket is securely fastened to the Option 11C and the grounding braid is connected to the card cage. |  |
|  | The NTRH0912 Multi IO cable is securely fastened to the Amphenol connector for the left hand slot of the 202i Card, and the amphenol drain wire is tightly connected a backplane grounding bolt. |  |
|  | 202i Installed in an Option 11C Mini or Avaya Communication Server 1000 Media Gateway. |  |
|  | The Option 11C Mini or CS 1000 has been audited and meets specifications. |  |
|  | The NTRH0912 Multi-IO cable is securely fastened to the Amphenol connector and routed neatly to the terminating points. |  |

General Installation

| Item | Description | Meets Specifications |
| --- | --- | --- |
|  | Cables associated with the CallPilot server must be installed in an orderly fashion, labeled on both ends, and fastened down completely on both ends. (This may minimize troubleshooting problems and tripping hazards.) [Major] |  |
|  | Ensure there are no unauthorized connections to the ELAN via the hub or switch. The ELAN carries call information between CallPilot and the switch. Any other network traffic could degrade CallPilot performance. [Major] |  |
|  | The ELAN and CLAN are on different subnets. [Critical] |  |
|  | The server has an operational mouse, terminal, and keyboard. Note: These devices should be available but not connected during normal operation. Connecting these devices could prohibit the installation of the front panel .[Recommended] |  |

Maintenance Items

| Item | Description | Meets Specifications |
| --- | --- | --- |
|  | If ELAN is connected to the M1 or CS 1000 via a layer 2 switch, ensure there is no packet loss. .[Major] |  |
|  | No critical CallPilot Manager alarms present. CallPilot Manager -> System ->Alarm Monitor[Critical] |  |
|  | No critical CallPilot Manager events present. CallPilot Manager -> System ->Event Browse[Critical] |  |
|  | All channels are in an “idle” or “active” state [Major] Maintenance Administration>Channel Monitor |  |
|  | All DSPs are in an “idle” or “active” state [Major] Maintenance Administration>Multimedia Monitor |  |
|  | An operational RAS modem is installed on the COM1 port of the server. [Recommended] |  |
|  | There are no mapped drives in the system. The CD-ROM drive is drive Z:[Major] |  |
|  | Server hard drive usage is below 90%[Critical] My Computer>Right mouse click on any drive>Go down to Properties |  |
|  | Available memory not below 4 MBytes[Critical] Start -> Programs -> Administrative Programs -> Performance |  |
|  | Processor time not more than 90% for any sustained period. [Critical] Start -> Programs -> Administrative Programs -> Performance (or Performance Monitor). |  |

Maintenance Items

| Item | Description | Meets Specifications |
| --- | --- | --- |
|  | Verify that the most up-to-date level CallPilot Server PEPs, and CallPilot Manager are installed. Ensure the associated patches are on the Meridian 1 or CS 1000. A visit to the www.avaya.com/support is recommended to obtain the most current information. [Recommended] |  |
|  | Ensure that both full and partial backups have been completing successfully. [Major] Schedules for full and partial backups to tape must be configured. (system -> backup/restore) Tapes must be stored in a secure place away from magnetic fields. Tape backup and tape rotation logs should be administered also. |  |
|  | All CallPilot CDs are available in case of the requirement to re-install the system. |  |
|  | Backups are not scheduled during “audits” (avoid 12 AM to 4AM) [Major] |  |
|  | No unauthorized third party software is installed and running on the server. [Major] (Symantec Anti Virus Corporate Edition 9 and 10.1, McAffee VirusScan 7.0 and 8.0 (but not version 8.5), TrendMicro OfficeScan versions 7 and 8, Computer Associates eTrust Antivirus 7.0 and 8.1are okay. Do not schedule during high traffic or audits. See P-2007-0101 Version 3) |  |
|  | Application Builder has no “locked” applications [Major] |  |
|  | PcAnywhere or RDC is configured and in “ready” mode on the server. (Required for remote support).[Major] |  |
|  | No Alarms present on the front of the server. |  |
|  | PBX has all required Patches for CallPilot |  |
|  | CallPilot software is compatible with CallPilot Manager and Application Builder |  |
|  | ### ELAN and CLAN interfaces have static IP addresses |  |
|  | ELAN interface does not have a default gateway |  |
|  | Remote Notification works |  |
|  | Check for DRWTSN32 and user.dmp logs. Make sure “Visual Notification” is not checked. |  |
|  | No shared folders exist on the CallPilot server |  |
|  | Comments: |  |

Product Bulletins for Vintage/Release Updates

| Product Bulletin | Affected Product | Part Description | Defective Part # | Replacement Part # | Reason for Change |
| --- | --- | --- | --- | --- | --- |
| 97030 | SL1-100 | Digital Line | NT8D02EA | NT8D02EB | M1 Option 11 through 81 unaffected |
| 97031 | Meridian 1 | XNET card | NT8D04BA, rel.3 after 9/96 | NT8D04BA, rel.4 | Loss of dial tone after power down of opposite network shelf; releases 1, 2, 3 before 9/96 not affected |
| 97043 | Option 11C |  | NTDK20AB, rel.6 | NTDK20AB, rel.6 | Clicking during established calls. Manufactured between 4-1-97 and 5-1-97 |
| 97051 | Meridian 1 | Call Processor | NT9D19AA NT9D19CA NT9D19HA | NT9D19AB NT9D19CB NT9D19HB | 68040 48MB/64MB/96MB |
| 97064 | Opt. 81C | QPC 441 | QPC441 prior to F | QPC 441 F or later | All 441s must be F or later |
| 98016 | NT5D12 | Dual Port PRI/DTI | Rls AA 4-10 | Rls AB 1-5 | Blows fuse on DCH/PRI daughter board |
| 98037 | Opt. 61C/81C | IODU/C NT5D61 | NT5D61AA Rel. 1-7 | NT5D61AB now rls 8 | Inadequately designed power trace |
| 98040 | All PBX IPE | XDAC card | NT7D16AA | NT7D16BA | DM to TLINK protocol enhancement |
| 98069 | Mini carrier remote | Local Interface (LMI) | NT5D64AA | Same 10/98 or later | Lockup –10/98 or earlier need terminator kit |
| 98069 | Mini carrier remote | Local Extender(LMX) | NT5D65AA | Same 10/98 or later | Lockup –10/98 or earlier need terminator kit |
| 98052 | Opt. 61C; 81C | FNET | NT1P61BA | NT7D16CA | Curtail PWR0013 & PWR0053 msgs. |
|  |  | FPEC | NT1P62BA | NT1P62CA | “ “ “ |
| 98053 | Carrier remote IPE | LCIC | NT7R51AA/AB | NT7R51AC | Y2K compliance issue w/MMI msgs |
|  |  | RCIC | NT7R52AA/AB | NT7R52AC | “ “ “ |
|  | Line side T-1 | Interface card | NT5D11 AA-AC | NT5D11AD | Y2K compliance issue w/MMI msgs |
|  |  | Interface (Opt.11) | NT5D14 AA-AB | NT5D14AC | “ “ “ |
|  | Fiber Remote IPE II | Fiber Interface | A0634488 | A0732456 | “ “ “ |
|  |  | Fiber Interface | A0634489 | A0732458 | “ “ “ |
|  |  | Fiber Interface | A0634490 | A0732527 | “ “ “ |
|  |  | Fiber Interface | A0634491 | A0732528 | “ “ “ |
| 98106 | Mmail | Meridian Mail Disk and Tape Drive Assembly Matrix |  |  |  |
| 99014 | Fiber Remote IPE II | Fiber Remote Multi-IPE -- FCC/IC Compliance |  |  |  |
| 99043 | Option 11C |  | NTAK02BB and prior vintages | NAK02BC | Card may sometime experience system synchronization errors |
| 99046 | Mini carrier remote | LMI Opt.21-81 LMX Opt.21-81 RMI LMI Opt.11 | NT5D64BA NT5D65BA NT5D67BA NT5D68BA NT5D69BA | NT5D64CA NT5D65CA NT5D67CA NT5D68CA NT5D69CA | The vintage of the Mini Carrier (MCR) interface cards was advanced to correct potential problems of short-term intermittent loss of digital set operation, in vintages and releases prior to the new CA vintage. |
| 99055 | Meridian 1 | DDP Card | NT5D12AC | NT5D12AD | The DDP card has been advanced in vintage to incorporate signaling enhancements necessary to support operation in global markets |
| 99070 | Conference | MICB | NT5D51AA | NT5D51AB | The Vintage of the Meridian Integrated (MICB) has been advanced to AB to improve reliability. |
| 99083 | Conference | MIRAN | NTAG36AA | NTAG36AB | The vintage of MIRAN card has been advanced. This modification resolve a possible issue with slight background noise on the A1 cross-connect channel when the MIRAN C: drive is active. This problem only occurs on cross-connect channel A1. |
| 99086 | Option 51C, 61C,81, 81C | X11 Release 25 Call Processor Changes/Memory Requirements |  |  |  |
| 99090 | Mmail Modular EC Option Systems | Meridian Mail Modular Option EC Backplane NT6P0102 | NT6P0102 rel 4 & 5 | NT6P0102 rel.6 | NT6P0102 rel.4 & 5 assemblies manufactured between October 1st, 1998 and September 25th, 1999, may encounter system failure and become out-of-service after a few months of successful operation in the field. |
| 99095 (see also 98093) | Fiber Remote | Fiber Remote Multi-IPE – New Backplane | A0759609 A0759612 A0759618 A0759620 | A0773056 A0773059 A0773054 A0773055 rls 3 or greater if daisy-chaining more than 2 units together for XSM communication is required. | Fiber Remote Multi-IPE Interface unit (FRMII) was modified to provide greater power surge immunity. |
| 99098 | Option 11C | NT8D09BA MWLC – Application with Option 11E/C Expansion Cabinet | NT8D09BA prior to release 5 | NT8D09BA rel.5 | The NT8D09BA Message Waiting Line Card (MWLC) fails to initialize if the clock signals are not present at the time power is applied to the card. This only occurs when the NT8D09BA is equipped in the Expansion Cabinet of the Option 11E and 11C systems using fiber optic cable connectivity to the main cab. |
| 99100 | Meridian 1 | Call Processor Portfolio Change / Memory Upgrade Kit Introductions |  |  |  |
| 99125 | Fiber | NT1P62 Fiber Controller: Vintage Update | NT1P62CA | NT1P62EA | The NT1P62 Fiber Peripheral Controller (FPEC) has been advanced in vintage to incorporate the firmware necessary to support the 24 port Digital Line Card (NTRD24). |
| 99124 | Meridian 1 | NT5D12 DDP: Vintage Update | NT5D12AD | NT5D12AF | The NT5D12 Dual Port Digital Trunk/Primary Rate Interface (DDP) has been advanced in vintage to incorporate modification as necessary to permit card identification. |
| 2000-004 | Mmail | Meridian Mail enhanced Card Option and Option 11 Battery Back Up Interaction | NT6R16AA prior to rel.6 | NT6R16AA rel.6 | If a power failure occurs to an AC-powered Opt.11 PBX that is using a battery backup, the Enhanced Card Option processor board will cease to function. When AC power is restored, the Enhanced Card Option must be manually rebooted for normal operation to resume. |
| 2000-006 | Mini- Carrier Remote | Mini-Carrier Remote and Controller – Four | NT5D64CA, NT5D65CA Release 1, 2 or 3 | NT5D64CA,NT5D65CA Release 4 & 5 | The release level of two Mini-Carrier Remote (MCR) cards has been advanced to include modifications required to improve certain operational aspects of the design. These design modifications will correct occasional problems experienced enabling the MCR cards when used with the M1 Controller – Four card NT8D01BC Release 10. |
| 2000-015 | Meridian 1 | X11 Release 25 Call Processor Changes / Memory Requirements |  |  |  |
| 2000-017 | Option 11C | DO NOT USE Call Pilot Mgate Card NTRB18CA in SLOT 10 of a Meridian 1 Option 11C | NTRB18CA rel.3 or prior releases | NTRB18CA rel. 4 | Option 11 Controller cards (NTDK20XX) and Option 11 Controller cards (NTDK20XX) and CallPilot Mgate cards (NTRB18CA) being damaged after the Mgate card is installed in slot 10 of a Meridian 1 Option 11. The controller card is damaged to the extent that the Option 11 stops call processing. |
| 2000-047 | Meridian 1 | Compatibility of NT5D12AA, AB, AE, AF DDP cards and X11 Rel.25 with Fiber Network Fabric (FNF) | Dual-port DTI/PRI (DDP) NT5D12AA, AB, AE & AF cards | NT5D12AC, AD or AG | Timing issue between the Fiber Network feature (FNF) and the dual port DTI/PRI (DPP) card when installed in the same Meridian 1 system. The problem severely impacts FNF feature operation and occurs on vintages AA, AB, AE and AF of the NT5D12 card. The problem has been corrected on Vintage AG of the NT5D12 card. |
| 2000-071 | Meridian 1 | Compatibility of the Dual-port DTI/PRI (DDP2) NT5D97AA and X11 Rel.25 with Fiber Network Fabric (FNF) | Dual-port DTI/PRI (DDP2) NT5D97AA card | NT5D97AB | Timing issue between the Fiber Network feature (FNF) and the dual port DTI/PRI (DPP2) card when installed in the same Meridian 1 system. The problem severely impacts FNF feature operation. The problem has been corrected on Vintage AB of the NT5D97 card. |
| 2000-075 | Meridian 1 | Multi-Frequency Receiver 911 PSAP Installations | NTAG26AA | NTAG26AB | A problem may occur with the NTAG26AA Multi-Frequency Receiver (XMFR) card in a 911 environment. 911 calls that terminate into a Meridian 1 system will intermittently hear a noise that lasts until ring-back tone is given. The noise is sometimes loud enough to cause the caller to hung-up and retry the call. Problem has been corrected on Vintage AB of the NTAG26 card. |
| 2000-077 | Meridian 1 | Digital Line Card | NT8D02GA rel.4 | NT8D02GA rel.4 | A limited number of NT8D02GA, rel.4 cards built between Feb.22, 1999 and March 15, 1999 may have an operational problem. Affected cards are identified with a faceplate label indicating warranty start date 5/99 or 6/99. |
| 2000-056 | Meridian 1 11C Mini | Digital Line Card | NTDK95BA Cabinet Inter-connect Cables | NTDK95BB cables | Cables are unreliable as far as connection ability. Order kit number NTDK89AA, which contains the “BB” cables. |
| 2000-092 | Meridian 1 | IODU/C Card | NT6D61AB rel 4 or 6 | NT6D61AB rel 7,8, 10, 12, or 15 | Some systems may experience a “lockup” or failure to communicate with the IODU/C card. |
| 2000-093 | Meridian 1 | MICB II | prior to 2.09 firmware | 2.09 firmware | Conferences drop. Reservations are lost. |
| 2001-004 | Meridian 1 | QSDI Paddleboard | NT8D41BA rel 1 to 5 | NT8D41BA rel 6 | Loss of data during large file/data transfers. Sites that may experience this condition may be locations with ACD MAX high-speed links or PMS links at hotels. |
| 2001-009 | Meridian 1 | Clock Controller | QPC471H | NTRB53AA | Noise on Meridian Mail when loops providing service to different nodes are spread amongst different FNF network groups in the PBX. This occurs when Overlay 30 swaps clocks during midnight routines. New clock card is not available until 2nd quarter 2002. Use mplr15446 to allow ovly 30 to run in MIDN but not swap clocks. |
| 2001-029 | Meridian Mail EC | MMP040 Processor | NT6R15AA prior to release 3 | NT6R15AA rel. 3 | AML errors due to clocking issues with MM12.14, 13.11, and 13.12. New card improves ACCESS link capabilities to SCCS. Replace the MMP040 only in the node containing the ACCESS link. |
| 2001-036 | Special Cables | Meridian 1 Option 81CPP II | Cables needed for network modules in other rows |  | See Bulletin for CPP and for 81C Motorola. |
| 2001-037 also 2003-088 | Meridian 1 | MSDL card NT6D80 | All MSDL prior to AC vintage | Use “AC” vintage | Clocking issues when placed in network group 6. Re-locate it or use NT6D80AC |
| 2001-072 | Meridian CPP II | BUG2285 |  |  | Insert PEP mplr15935 |
| 2001-078 | Meridian Mail | Voice Processor 4 or 8 | NT6P04AA or 08AA Rls 7 5/01 to 9/01 |  | Noisy Channels. Replace with rls 8 cards. |
| 2002-018 PAA-2003-1548 Rev2 | Meridian 1 | Message Waiting Line Card | NT8D09BA rls 6 and lower NT8D09BA rls 1 to 8 | NT8D09BA rls 7 NT8D09BA rls 10 | Noise on ports. Symptoms clear when card is reseated or powered down. Send cards in for retrofit to rls. 7 Noise on ports. Retrofit good until June 2005 |
| 2002-024 | Meridian 1 CPP II | Memory upgrade | NT4N19AA | NT4N19AA | This memory upgrade kit is required for all CPP II systems wishing to upgrade to X11 25.40 running 6 or more network groups. Upg to 256MB |
| 2002-025 | SCCS 4.0 Client | Client Software | 4.0 Client software for WIN2000 or NT 4.0 | Use Pentium 3 systems | Client software installation will freeze on a P4 system. See workaround for Win2000 & NT 4.0 Clients only W95 & W98 not supported. |
| 2002-020 | Meridian 1 CPP | MMDU | NT4N43AA/BA | NT4N43CA- both sides must be “CA” vintage units !!!! | Drive failures- Replace only if a site has particular drive related issues only. |
| 2002-1596 | Meridian 1 | Clock Controller | QPC471 | NTRB53AA | Improved clock synchronization on FNF systems |
| 2002-1600 | Meridian 1 | ITG Trunk or Line | NT0961AA/0966AA NTVQ55AA/ZC80AA |  | Update firmware to v5.7 to fix a call swap issue. |
| 2002-1658 | Meridian 1 | FIJI Card | NTRB33AA rls 11 or greater | NTRB33AC or AD | Updated firmware to work with NTRB53AA clock card. |
| NF08-2003 | Meridian 1 | M3904 & M3905 | Firmware prior to 8.5 | Install firmware 8.5 | Need loadware PEP’s mplr17093 to mplr17096 for M3904 and mplr17097 to mplr17100 for M3905 sets. |
| 96063 | Meridian Mail | MMP040 Processor for Mod. Option & EC | NT6P97AA-EC NT4R45AA-Mod Opt | NT6P97AA rls 13 or above NT4R45AA rls 10 or later | To fix parity error 17 during Boot ROM diagnostics |
| PAA2003-0321 | Symposium | Server | Servers on 3.0 or above | MS03-026 MS03-039 | Microsoft security fixes for RPC errors |
| PAA2003-0322 | CallPilot | Server | Servers on 2.0 or 2.02 | PEP’s 47S and 48C | To fix backup and restore problems |
| Stop Ship | Remote Office for Meridian 1 | NTDR76BB-9110 NTDR77BB-9115 | A0505173 A0505174 | Use latest units later than 1/20/04 | Bad FET when loop current is up around 40ma |
| 2003-340 | Meridian Mail | NT6P97AA MMP040 | NT6P97AA rls 18 & 19 Shipped 1/98 to 7/99 | NT6P97AA rls 20 or 22 | Intermittent reboots with “Unknown daughterboard interrupt” |
| PAA2003-0435 | CallPilot 2.02 (2.01.27.05) | Servers |  | All installed servers | Install most current PEP’s: |
| PAA 2004-0055 | Meridian 1 | MMDU | NT4N43CA rls 5 | NT4N43CA rls 3 or 4 | A release 25.40b disk dated before 12/19/2003 cannot be read by CD-ROM drive. The fix is to use a later CD-ROM |
| P-2004- | All Succession | Signaling Server | NTDU27AA | NTDU27AA w/512MB memory | If over 5,000 IP phones are used you must upgrade the memory to 512 MB using the NTDU80CA Upgrade Kit. |
| P-2004-0058 Rev 2 | All Succession | Signaling Server | NTDU27AA | NTDU27AA w/ new loadware | Use loadware 2.11.03 for latest fixes |
| PAA-2004-224 | Meridian 1 | FIJI Card | NTRB33AD rls 7 NTRB33AD rls 8 | NTRB33AD rls 9 NTRB33AD rls 10 | Incorrect loadware installed at factory on rls 7 and 8. |
| PAA-2004-412 | Meridian 1/Succession | RLC, 9110,9115, 9150 | Firmware | Updated firmware | Memory leak on affected hardware. Upgrade to 1.5.2 firmware. |
| P-2005-0013 | Succession/ M1 | M3900 sets | Old set firmware | M3902- 8.4 M3903- 8.7 M3904/M3905- 8.9 | New firmware updates |
| P-2005-0038 | CallPilot 1002 trp rack servers | SCSI power cable | N0029330 on any system before 2/2005 | New N0029330 | Order new SCSI power cable by end of 9/2005. Old cable has no over current protection. |
| P-2005-054 | Succession | IP phone firmware |  | 0604D88 firmware | IP enhancements |
| P-2005-0070 | Succession | MIRAM III | 3.0M | 3.0P.03 | To implement BUG fixes |
| P-2005-0098 | Meridian 1/Succession | IP Trunk 3.01 ITG Trunk 2.1 | 3.01 or 2.1 | 3.01.60 | To derive security enhancements |
| P-2005-0129 | Meridian 1/Succession | Message Waiting Line Card | NT8D09BB or below | NT8D09CA | Electromagnetic coupling improvements/higher voice quality |
| P2005-0131 | Meridian Option 61C/Succession 1000SG | Clock to clock cables | NT8D75 | Use NT8D75DA on AC powered systems | Fixes clock switching problems |